# Pixelink Bluetooth SDK SRS

## January 25, 2005

## SCOPE

The Pixelink Bluetooth SDK provides application developers with the means to retrieve image files from a variety of Bluetooth-enabled mobile devices.  It does so by providing support for retrieving images via three different GOEP-based profiles: Object Push Profile, File Transfer Profile and Basic Printing Profile.

## DESIGN CONSTRAINTS

| CONSTRAINT.LANG | The API visible to application developers shall consist of a set of C# classes. Source: Customer Requirement: Required, Mandatory by Customer |
| --- | --- |

| CONSTRAINT.API | The API visible to application developers shall conform to the API shown in TBD. Source: Customer Requirement: Required, Mandatory by Customer |
| --- | --- |

| CONSTRAINT.PLATFORM | The system shall run on a Windows XP SP2 personal computer. Source: Customer Requirement: Required, Mandatory by Customer |
| --- | --- |

| CONSTRAINT.HARDWARE | The system shall be compatible with the 3Com Bluetooth USB Adaptor model number 3CREB96B (TBD) Source: Customer Requirement: Required, Mandatory by Customer |
| --- | --- |

###### PRODUCT FEATURES

| PROFILE.ACTIVATION | The system shall provide a means to activate the Bluetooth Protocol Stack. When activated, service records for the BPP, FTP, and OPP profiles shall be inserted into the SDDB. If it is not already so, the Bluetooth adaptor shall be put into “General Discoverable” mode. Source: Customer provided API Requirement: Required, Mandatory by Customer |
| --- | --- |

| PROFILE.DEACTIVATION | The system shall provide a means to deactivate the Bluetooth Protocol Stack. When deactivated, the service records shall be removed from the SDDB and all open connections shall be closed. Source: Customer provided API Requirement: Required, Mandatory by Customer |
| --- | --- |

| PROFILE.CONCURRENCY. LISTEN | The system shall allow for any combination of the three profiles to be active concurrently without interfering with each other. Source: Pixelink Requirement: Required |
| --- | --- |

| SECURITY.FILESYSTEM. LOCAL | The system shall not allow mobile devices to browse the local file system. Source: Customer Requirement: Required, Mandatory by Customer |
| --- | --- |

| SECURITY.FILESYSTEM. REMOTE | The system shall not manipulate the file system of any mobile device. Source: Customer Requirement: Required, Mandatory by Customer |
| --- | --- |

| SECURITY.FILETYPES | The system shall only receive files of MIME type image/jpeg or text/xhtml format. Source: Customer Requirement: Required, Mandatory by Customer |
| --- | --- |

| SECURITY.AUTH | The system shall permit mobile devices to connect using shared-key authentication. Source: Customer Requirement: Required |
| --- | --- |

# Generic OBEX Server Requirements Specification

SCOPE

The Generic Object Exchange Profile provides support for applications that require object transport capabilities. The fundamental features of the OBEX profile are the abilities to pull and push data objects. OBEX interacts above the Serial Port Profile and beneath high level profiles, including Object Push Profile, File Transfer Profile, and Basic Printing Profile.

FEATURE OVERVIEW

- Establishing a Connection

- Receiving  a Data Object

- Responding to Request for a Data Object

ESTABLISHING AN OBJECT EXCHANGE SESSION

This feature establishes a connection between the client and server. Objects cannot be exchanged before the session is established.

| OBEX.CONN.OPEN | The server shall support a connection without authentication. Prior to connection, the server shall be in a discoverable state, and a link will be established to communicate with the client device. Sources: Bluetooth Specification, vol. 2, pg. 320, 324-326 Fields in Connection Request Message: Name; Value; Status. Opcode for CONNECT; 0x80; Mandatory. Request Packet Length; Mandatory. OBEX Version Number; Mandatory. Flags; Mandatory. Max OBEX Packet Length; Mandatory. Fields in Connection Response Message: Name; Value; Status. Response code; 0xA0 for success varies for error; Mandatory. Response Packet Length; Mandatory. OBEX Version Number; Mandatory. Flags; Mandatory. Max OBEX Packet Length; Mandatory. Requirement: Required, Mandatory Bluetooth Specification |
| --- | --- |

| OBEX.CONN.AUTH | The server shall support connections from devices with shared-key authentication. Sources: (Bluetooth Specification, vol. 2, pg. 324 & 327) Requirement: Not Required, Mandatory by Specification, Not Required by Customer |
| --- | --- |

| OBEX.DISCONNECT | (TBD: In an IrDA specification ) Requirement: ??? |
| --- | --- |

| OBEX.MULTI | The server shall permit concurrent connections from multiple clients. Sources: Bluetooth Specification, vol. 2, pg. 320 Requirement: Not Required, Optional by Bluetooth Specification |
| --- | --- |

RECEIVING A DATA OBJECT

This feature is used to transfer data from client to server.

| OBEX.PUT | The server shall accept PUT requests from a client, containing byte arrays that span one or more OBEX packets. Messages shall be held in queue for the appropriate profile. Source: (Bluetooth Specification, vol. 2, pg. 330) Fields in PUT Request Message: Name; Value; Status. Opcode for PUT; 0x02 or 0x82; Mandatory. Packet Length; Mandatory. Name; Mandatory. Body / End Body; Mandatory. Requirement: Required, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

| OBEX.PUT.FOLDER | The server shall accept PUT requests from client, containing recursive folders. Source: (Bluetooth Specification, vol. 2, pg. 337 – 338) Requirement: Required, Mandatory by Specification, Optional by Customer |
| --- | --- |

| OBEX.PUT.REPLY | The server shall respond to successful PUT requests with a single packet confirmation. Source: (Bluetooth Specification, vol. 2, pg. 330) Fields in PUT Response Message: Name; Value; Status. Response Code for PUT; 0x90 or 0xA0; Mandatory. Packet Length; Mandatory. Requirement: Required, Mandatory by Specification |
| --- | --- |

RESPONDING TO A REQUEST FOR A DATA OBJECT

This feature is used to transfer data from server to client.

| OBEX.GET | The server shall push GET requests to a client, containing byte arrays that span one or more OBEX packets. Source: (Bluetooth Specification, vol. 2, pg. 331 - 332) See Bluetooth Specification vol. 2, Part K:10, Tables 5.11 for required fields. Requirement: Not Required, Mandatory by Specification |
| --- | --- |

| OBEX.GET.FOLDER | The server shall push GET requests to a client, containing recursive folders. Source: (Bluetooth Specification, vol. 2, pg. 337 – 338) Requirement: Not Required, Mandatory by Specification, Not Required by Customer |
| --- | --- |

| OBEX.GET.REPLY | Source: (Bluetooth Specification, vol. 2, pg. 331 - 332) See Bluetooth Specification vol. 2, Part K:10, Table 5.12 for required fields. Requirement: Not Required, Mandatory by Specification |
| --- | --- |

| OBEX.TIMEOUT | The server shall wait 30 seconds for a response to requests before timing out. (TBD …. Is retransmission required?) Sources: Bluetooth Specification, vol. 2, pg. 323 Requirement: Required, Optional by Bluetooth Specification |
| --- | --- |

SHARED FUNCTIONALITY

| OBEX.ABORT | (TBD: Referred to, but not defined in the spec ….) Requirement: ???, Mandatory by Specification |
| --- | --- |

INTEROPERABILITY WITH PROFILES USING OBEX

Profiles using OBEX:

Operation no. OBEX Operation

Connect

Disconnect

Put

Get

Abort

SetPath

# Object Push Profile (OPP) Server Requirements

The Object Push Profile provides support for transferring objects between two Bluetooth devices. The OPP server will limit its capability to receiving objects from a client. This profile uses the Generic Object Exchange profile (OBEX) as a base profile to define the interoperability requirements for the protocols needed by the applications.

RECEIVING AN OBJECT FROM A CLIENT

| OPP.PUSH | The OPP Server shall allow clients to use the push operation to upload files. Source: OPP Specification Requirement: Required, Mandatory by OPP Specification, Mandatory by Customer |
| --- | --- |

| OPP.PUSH.APPROVAL | The OPP Server shall provide the Kiosk Application with a way to accept or reject client requests to upload files. Source: Customer provided API Requirement: Required, Mandatory by Customer |
| --- | --- |

| OPP.PUSH.FILE_LOCATION | The OPP Server shall provide the Kiosk Application with a way to set the location in the file system that files are uploaded to. Source: Customer provided API Requirement: Required, Mandatory by Customer |
| --- | --- |

| OPP.PUSH.FILE_RETRIEVAL | The OPP Server shall provide the Kiosk Application with a way to set retrieve uploaded files. Source: Customer provided API Requirement: Required, Mandatory by Customer |
| --- | --- |

| OPP.PUSH.SECURITY | The OPP Server shall only accept files that have image/jpg mime type. TBD is this really the only format: question for Jack. If a client tries to push a file that does not have a valid extension the OPP Server shall respond with an appropriate error message. This is not standard OPP but specific to the Kodak application. Source: Kodak, find actual source (email?) Requirement: Required, Mandatory by Customer |
| --- | --- |

RESPONDING TO A CLIENT PULL REQUEST

| OPP.PULL | The OPP Server will respond to Pull requests with an appropriate error message. Source: OPP Specification 4.3 Requirement: Required, Mandatory by OPP Specification, Mandatory by Customer |
| --- | --- |

| OPP.EXCHANGE | The OPP Server will respond to Business Card Exchange requests with an appropriate error message. Source: OPP Specification 4.4 Requirement: Required, Mandatory by Customer, Mandatory by OPP Specification |
| --- | --- |

OBEX INTEROPERABILITY

| OPP.OBEX.HEADERS | The following OBEX headers are required for the OPP. These are the headers that are marked as Mandatory in the OPP Spec. We may want to support some of the Optional headers as well TBD. - Name - Type (this one is Optional but is recommended by the OPP Spec 5.5) - Length - Body - End of Body Source: OPP Specification 5.2 Requirement: Required, Mandatory by OPP Specification |
| --- | --- |

# FTP Server Requirements

SCOPE

The File Transfer Profile provides support for transferring objects (files and folders) between two Bluetooth devices, browsing the file system of a Bluetooth device, and deleting files and creating new folders on another Bluetooth device. The FTP server will limit its capability to receiving files from a client. This profile uses the Generic Object Exchange profile (OBEX) as a base profile to define the interoperability requirements for the protocols needed by the applications.

RECEIVING A FILE

| FTP.PUSH | The FTP Server shall support receiving files from a client. Source: FTP Specification 4.3 Requirement: Required, Mandatory by FTP Specification, Mandatory by Customer |
| --- | --- |

| FTP.PUSH.FILE_TYPE | The FTP Server shall only accept valid image files with ‘image/jpeg’ MIME type. If a client tries to push an invalid file the FTP Server shall respond with an appropriate error message. Source: API received by customer Requirement: Required, Mandatory by Customer |
| --- | --- |

RESPONDING TO CLIENT FOLDER BROWSE REQUESTS

| FTP.FOLDER_BROWSE | The FTP Server shall reply with an Unauthorized or Forbidden response to refuse to disclose the contents of the root folder. Source: FTP Specification 4.2 Requirement: Required, Mandatory by FTP Specification, Not Required by Customer |
| --- | --- |

RECEIVING A FOLDER

| FTP.FOLDER_RECEIVE | Server shall accept folders. Source: ? Requirement: Optional, Optional by FTP Specification, Optional by Customer |
| --- | --- |

RESPONDING TO CLIENT PULL REQUESTS

| FTP. PULL | The FTP Server shall respond with an appropriate error message if the client tries to pull any files from the Server. Source: FTP Specification 4.3 Requirement: Required, Mandatory by FTP Specification, Mandatory by Customer |
| --- | --- |

RESPONDING TO CLIENT REQUESTS TO MANIPULATE THE FILE SYSTEM

| FTP. FILE_MANIP | The FTP Server shall respond with an appropriate error code if the client tries to delete a file or create a new folder. TBD Source: FTP Specification 4.4 Requirement: Optional, Optional by FTP Specification |
| --- | --- |

OBEX INTEROPERABILITY

| FTP.OBEX.HEADER | The FTP Server shall support the following OBEX headers: (Only the mandatory headers are listed) - Name - Type - Length - Target - Body - End of Body - Who - Connection ID - Authenticate Challenge - Authenticate Response Source: FTP Specification 5.2 Requirement: Required, Mandatory by FTP Specification |
| --- | --- |

# BPP Requirements

### SCOPE

The Basic Printing Profile is a Bluetooth profile that will be used specifically for transferring objects that will be printed (as opposed to the OPP and FTP profiles which were created only to transfer objects and files), and commands to print them in certain ways.

There is only a subset of the profile that the kiosk application will need to perform.  This subset does not encompass the mandatory functionality within the profile specification, so some mandatory commands will be taken in, parsed and neutralized (maybe sending back null data or just plain not executing).

The full profile is specified in file BPP_SPEC_D10.pdf.

#### ACCEPT ONLY IMAGE DATA

| BPP.ACCEPT.MIME | The system shall accept data with a MIME type of image/jpeg for printing Source: Customer Requirement: Required, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

| BPP.ACCEP.XHTML | The system shall accept data with a type of XHTML Print if it contains inline image data for printing. Source: Customer Requirement: Optional, Optional by Specification Acceptance Criteria: |
| --- | --- |

| BPP.ACCEPT.TEXT | The system shall accept any basic text that is sent for printing. Source: Customer Requirement: Optional, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

#### MECHANISMS

| Mechanism Name | Required |
| --- | --- |
| Simple Push | Required |
| Job Based Transfer | Required |
| Print by Reference | Optional |
| Reflected User Interface | Not Required |

#### SIMPLE PUSH

The simple push model is a “best effort to print” exchange.  No provisions shall be made for the sending device and the printer’s current settings will be used.  The sender only sends an object with faith that it will be printed.  The sender can send an object, and if reference printing is implemented, a reference may be sent.

| BPP.SPUSH.INLINE | The system shall accept a supported type of data to be printed through the simple push mechanism. If there is an error, the system shall respond with an appropriate status code. Source: BPP spec pg. <<FILL IN>> See BPP Specification section 16.2 for status codes Requirement: Required, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

| BPP.SPUSH.REF | The system shall receive referenced content for printing. Upon receiving a simple push with referenced data, the system shall open a second channel, where it will act as client, to receive the referenced content for printing from the sending device. If there is an error, the system shall respond with an appropriate status code. Source: BPP spec pg. <<FILL IN>> See BPP Specification section 16.2 for status codes Requirement: Optional, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

JOB BASED

Job Based printing is a more robust mechanism than Simple Push.  Here is the basic Job Based print scenario:

- User sends a Create Job SOAP message to the system.

- The system returns with a job ID to reference the newly created job.

- The user sends a Send Document SOAP message to the system with the data he/she wishes to print.

- At any time the user can send a Cancel Job request with the reference job ID to cancel a print job.

These are the states and allowable actions of the Job Based printing mechanism:

Of the mandatory set of operations to support, this system will execute (or return with appropriate status code) the following Job Based commands:

| BPP.JOB.CREATE | The system shall accept and execute the Create Job command and create a print job with a selection of the parameter attributes (e.g. Copies, quality, etc.). All parameters will be discarded except the copies parameter which will be functional. Source: BPP spec pg. 40 Requirement: Required, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

| BPP.JOB.ATTR | The system shall execute the Get Job Attributes command and return the desired attributes of a print job given a job ID. All attributes listed in the specification will be supported. Source: BPP spec pg. 44 Requirement: Required, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

| BPP.JOB.PATTR | The system shall execute the Get Printer Attributes command and return selected attributes of a printer. Source: BPP spec pg. 33-39 Requirement: Required, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

| BPP.JOB.CANCEL | The system shall cancel a job with the specified job ID upon receiving a Cancel Job command. Source: BPP spec pg. 46 Requirement: Required, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

| BPP.JOB.GETEV | The system shall execute the Get Event command and set up a new RFCOMM channel for notification of any change in printer status with reference to a specified job ID to be sent to the client (status such as printer attributes at times, percent printed, any printer event) Source: BPP spec pg. <<FILL IN>> Requirement: Required, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

| BPP.JOB.SENDDOC | The system shall be able to receive the Send Document command to receive print information associated with a parameter job ID. Source: BPP spec pg. 43 Requirement: Required, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

| BPP.JOB.GETREFD | The system shall execute a get referenced content command over a second RFCOMM channel to receive any image data that has been sent as a reference. Source: BPP spec pg. 48 Requirement: Optional, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

#### OBEX HEADERS

| BPP.OBEXHEAD | The system shall use the Type, Body, Description, Name and Application Parameters headers as described in the specification. Source: BPP spec pg. 99 Requirement: Required, Mandatory by Specification Acceptance Criteria: |
| --- | --- |

##### GLOSSARY

BPP	Basic Printing Profile.  Used to push image files from one Bluetooth-enabled mobile device to another and manage remote print queues.  Relies on GOEP.

GOEP	Generic Object Exchange Profile.  Used to transfer objects (such as images, vcards or vcalendars) from one device to another.  Used to implement BPP, FTP and OPP.

FTP	File Transfer Profile.  Used to push or pull image files from one Bluetooth-enabled mobile device to another as well as browse the remote file system. Relies on GOEP.

OBEX	Object Exchange.  See GOEP.

OPP	Object Push Profile.  Used to blindly push an image file from one Bluetooth-enabled mobile device to another. Relies on GOEP.

SDDB	Service Discovery Database.  Houses service records describing the various Bluetooth services provided by this device.

Service Attribute	A data member of a service record that indicates a certain characteristic of that service.  Service attributes of requested services are compared to service attributes of available services to determine a match.

Service Record	An entry in the SDDB containing service attributes of a particular service (such as BPP, FTP or OPP).